# LCP Windows Build Guide:

# Toolchain Installation Requirements

- Visual Studio code:

  - https://code.visualstudio.com/

- GNU ARM Embedded Toolchain:

  - https://developer.arm.com/Tools%20and%20Software/GNU%20Toolchain

- CMake For Windows:

  - https://cmake.org/download/

- Ninja CMake Backend:

  - https://ninja-build.org/

- Segger J-Link Tools:

  - https://www.segger.com/downloads/jlink/

- AmbiqSuiteSDK:

  - Clone the repo:  https://github.com/sparkfun/AmbiqSuiteSDK

# Ensure Toolchain is in the system Path:

Go to “Edit the system environment variables” and ensure that the following entries are in the user “Path” environment variable:

- C:\Program Files (x86)\Arm GNU Toolchain arm-none-eabi\14.2 rel1\bin

- C:\Tools\ninja-win

- C:\Program Files\SEGGER\JLink_V860

Note: “14.2 rel1” may be different, check your installation and use the correct version path.

Note: “C:\Tools\ninja-win” can be wherever the ninja binary is located in the system.

Note: “C:\Program Files\SEGGER\JLink_V860” may be different, check your installation and use the correct version path.

# Hardware Required:

- Tag Connect Plug o’nails: TC2030-CTX

- SEGGER J-Link EDU Mini V2

- USB to TTL serial adapter:  FT232RL

# Build Commands:

- In the project directory C:\…\EDD-LCP_Control\Firmware\LCP_Control create a new directory called “build” and change to that directory:

  - mkdir build

  - cd build

- In the ‘build’ directory, use CMake to get the project ready for building with the toolchain:

  - cmake .. -DCMAKE_TOOLCHAIN_FILE=”../arm-gcc-toolchain.cmake” -G “Ninja”

- With the J-link connected, power on the board and use Ninja to build the project and flash it to the hardware:

  - ninja flash_jlink

- Alternative flash using RS232 to TTL (only works if there is no error state on control board, i.e. not the first flash and logger has the correct pressure profiles).

  - Ninja bootload_asb